List of Safety Requirements

| N° | ID | Name | Description | Kind | ASIL | Time constraint | Physical constraint | Comment | Traced FTA Events | Status | Related Goals | Contributions | Contributes To | Allocations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SR001 | [NOTIFY] Notify the driver / passengers about presense of unintended HV or leakage |  | FUNCTIONAL | A |  |  |  | no traced events | PROPOSED | • G003 (ASIL A) | • SR227 (ASIL A) |  | • Master Controller |
| 2 | SR003 | Prevent battery from operating outside temperature SOA |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR035 (ASIL B) |  |  |
| 3 | SR004 | Prevent battery from operating outside voltage SOA |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR037 (ASIL B) |  |  |
| 4 | SR005 | Prevent battery from operating outside current SOA |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR038 (ASIL B) |  |  |
| 5 | SR006 | [DETECTION ¦ TIMING] Detect impending or occurance of gassing or TR within xxx seconds of first occurance | The BMS shall continuously monitor battery status to detect / predict an impending gassing or thermal runaway event. | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G002 (ASIL A) | • SR017 (ASIL B) | • SR246 (ASIL B) | • Master Controller (ASIL B) |
| 6 | SR008 | [DETECT] Continuously monitor individual series cell voltages |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR021 (ASIL B) | • SR004 (ASIL B) |  |
| 7 | SR010 | [DEGR ¦ TR ¦ SS] Temporarily disconnect the battery if fault has been detected |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR254 (ASIL B) | • SR003 (ASIL B) |  |
| 8 | SR011 | [DETECT] Continuously monitor battery temperature |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR009 (ASIL B) | • SR003 (ASIL B) |  |
| 9 | SR012 | [AVOID] Provide battery performance limits to connected loads and sources |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR031 (ASIL B) |  |  |
| 10 | SR013 | [NOTIFY] Notify the driver and passengers about fault and warning condition related to battery or cells |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR035 (ASIL B) | • SR003 (ASIL B) |  |
| 11 | SR014 | [DEGR ¦ TR ¦ SS] Disconnect the battery if over-current fault has been detected | The BMS shall disconnect the battery within a stipulated time period of occurrance of the gassing event. | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR266 (ASIL B) | • SR005 (ASIL B) |  |
| 12 | SR015 | [DETECT] Continuously monitor battery current |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR028 (ASIL B) | • SR005 (ASIL B) |  |
| 13 | SR018 | [TIMING ¦ TOL ¦ DETECT] Detect HV exposure fault within xxx seconds of its occurrance |  | FUNCTIONAL | A |  |  |  | no traced events | PROPOSED | • G003 (ASIL A) | • SR000 (ASIL A) |  | • Master Controller |
| 13.1 | SR335 |  |  | UNSPECIFIED | A |  |  |  | no traced events | PROPOSED | • G003 (ASIL A) |  |  |  |
| 14 | SR019 | [DEGR ¦ SS] Permanent disconnect the battery from the external HV bus | While the goal is derived from ASIL A requirements, it is assigned ASIL B because: the functional implementation of such requirement might (or might not) be through same mechanisms which implement other such requirements, such as SR040. | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G002 (ASIL A) | • SR232 (ASIL B) |  |  |
| 15 | SR049 | Protect battery from accelerated aging and wear | Balancing of cells is required for optimal battery operation during charge and discharge cycles. An unbalanced battery will not contain the energy designed for and will experience accelerated aging and increased ware. | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR008 (ASIL B) |  |  |
| 16 | SR051 | Balance the battery | The BMS shall balance the unbalanced cells in the battery when cell voltage imbalance between the cell with highest voltage and the one with lowext voltage exceeds xxx mV. | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR047 (ASIL B) | • SR049 (ASIL B) |  |
| 17 | SR062 | Maintain proper BMS operation | The BMS shall include methods for ensuring the intended operation | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED |  | • SR130 (ASIL B) |  |  |
| 18 | SR073 | [DETECT] Continuously monitor battery voltage |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR050 (ASIL B) | • SR004 (ASIL B) |  |
| 19 | SR123 | Protect BMS against internal communication errors | The BMS shall implement detection and correction mechanisms for internal communication failures between various hardware units on the board. | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR270 (ASIL B) | • SR249 (ASIL B) |  |
| 20 | SR248 | Prevent BMS from operating outside SOA |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR250 (ASIL B) |  |  |
| 21 | SR249 | [AVOID ¦ DET] Continuously monitor BMS state for internal faults |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR118 (ASIL B) | • SR248 (ASIL B) |  |
| 22 | SR250 | [SS] Transition system into safe state upon encountering critical faults |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR101 (ASIL B) | • SR248 (ASIL B) |  |
| 23 | SR251 | [NOTIFY] the driver against BMS faults or warnings |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) | • SR087 (ASIL B) | • SR248 (ASIL B) |  |
| 24 | SR267 | [NOTIFY] Notify driver about gassing or thermal runaway event within xxx seconds of detection |  | FUNCTIONAL | B |  |  |  | no traced events | PROPOSED | • G002 (ASIL A) | • SR246 (ASIL B) | • SR251 (ASIL B) |  |
| 25 | SR290 | Ensure BMS components operate in their safe operating temperature limits |  | TECHNICAL | B |  |  |  | no traced events | PROPOSED | • G004 (ASIL B) |  | • SR248 (ASIL B) |  |