work product ISO/DIS 26262-3 clause 5.5

| N° | Resource Title | Resource URL |
| --- | --- | --- |
|  | README.md | Re_Work_FUSA_BMS_ASIL_C_2W_11_08/ |
|  | LICENSE.txt | Re_Work_FUSA_BMS_ASIL_C_2W_11_08/LI |

| Name | Type | Allocated Functions/Activities | Contained by | Connected to |
| --- | --- | --- | --- | --- |

| ID | Name | Description | Allocation |
| --- | --- | --- | --- |
| F-20 | Ensure Battery is operating in SOA. | Ensures the Safe Power Delivery to the Vehicle |  |
| F-19 | Measures Battery Insulation Resistance |  |  |
| F-18 | Communicate with externally connected devices. | Battery state information, temperature, voltages, fault conditions, etc. |  |
| F-16 | Perform State Estimations (SOC, SOH, SOP) | (OV/UV/OC/SCP/OT/UT) |  |
| F188 | Perform HVIL Integrity Check |  |  |
| F-15 | Measure battery temperature |  |  |
| F-14 | Measure battery voltage |  |  |
| F-22 | Measure battery current |  |  |
| F-21 | Detect overtemperature condition | Prevent battery from entering and staying in overtemperature condition |  |
| F-13 | Detect undertemperature condition |  |  |
| F-12 | Detect overvoltage condition |  |  |
| F-11 | Detect undervoltage condition |  |  |
| F-10 | Detect open cell connection |  |  |
| F-9 | Detect overcurrent condition |  |  |
| F-8 | Detect thermal runaway condition |  |  |
| F-6 | Measure FET temperature |  |  |
| F-4 | Notify user about critical fault conditions | Notify the user about hazardous / life threatening conditions such as thermal runaway, fire, gassing / venting, etc. |  |
| F-3 | Communicate with charger | BMS must communicate with charger to ensure charger always provides power to battery within its safe operating region | Master Controller |
| F-2 | Wakeup BMS on external command | Wake up BMS from sleep mode / power down mode upon external command / request. |  |
| F-1 | Estimate internal resistance | IR estimation of the battery or cells |  |
| F117 | Measure insulation resistance | Measure insulation resistance between the Pack+ and Bat- |  |
| F-17 | Balance battery | BMS must balance an imbalanced battery pack |  |
| F-7 | Control FETs | Control charge /discharge path mosfets |  |
| F-24 | Send / receive data on CAN |  |  |
| F-25 | Estimate battery SOC |  |  |
| F-30 | Detect open cell connection |  |  |
| F-31 | Notify user about critical faults |  |  |
| F-32 | Perform BMS self diagnostics |  |  |
| F-33 | Communicate with charger |  |  |
| F-34 | Wakeup BMS on external command |  |  |
| F-35 | Emergency disconnect |  |  |
| F-39 | Estimate battery SOH |  |  |
| F-41 | Perform precharge sequence |  |  |
| F-54 | Measure battery internal resistance |  |  |
| F-49 | Measure shunt temperature |  |  |
| F-55 | Measure link voltage |  |  |
| F111 | Detect current based faults. |  |  |
| F-47 | Detect cell voltage based faults |  |  |
| F-40 | Detect temperature based faults |  |  |
| F192 | Estimate battery SOP |  |  |

| ID | Name | Description | Allocation |
| --- | --- | --- | --- |
| F193 | Detect Thermal Runaway |  |  |
| F195 | Get data from Power Distribution Section (PDU) |  |  |
| F-52 | Control current flow through HV lines |  |  |
| F-48 | Measure battery current |  |  |
| F112 | Detect short-circuit condition |  |  |
| F-36 | Control precharge switch |  |  |
| F-51 | Control HV power path switch |  |  |
| F-56 | Control balancing circuit |  |  |
| F-59 | Provide temperature and voltage information to master |  |  |
| F-60 | Provide fault information to master |  |  |
| F-57 | Measure cell voltages |  |  |
| F-58 | Measure battery temperature |  |  |
| F-23 | Balance battery | MCU implements algorithms to balance the battery |  |
| F-42 | Measure battery voltage |  |  |
| F-43 | Supply power for low voltage analog systems |  |  |
| F-44 | Monitor input power quality |  |  |
| F-45 | Monitor output power quality |  |  |
| F-61 | Provide power supply quality information to Master |  |  |
| F-53 | Measure insulation resistance |  |  |
| F115 | Collect cell voltages |  |  |